Hybrid Local + Cloud IoT System

ESP-NOW Gateway with MQTT over TLS

Project Documentation – Track B (Advanced)

Hardware: 2 × ESP32 DevKit

# 1. Project Overview

This project is a complete multi-node IoT system built on two ESP32 boards. A battery-powered Sensor Node reads temperature and humidity and sends the data locally over ESP-NOW to a main Gateway. The Gateway aggregates the data, makes a local decision (LED and buzzer alert), displays the values on an OLED screen, and securely forwards them to the cloud through MQTT over TLS (HiveMQ Cloud).

# 2. Objectives

- Use low-power local communication (ESP-NOW) combined with Deep Sleep on the sensor node.

- Build a gateway that handles two different networking protocols (ESP-NOW and WiFi/MQTT) at the same time.

- Make local decisions without depending on internet connectivity.

- Monitor the gateway status using MQTT Last Will and Testament (LWT) and birth messages.

# 3. System Architecture

[Board A: Sensor Node] --ESP-NOW--> [Board B: Gateway] --WiFi + MQTT/TLS (8883)--> [HiveMQ Cloud]

DHT11 + Deep Sleep (30 s)           OLED + LED + Buzzer                          MQTT Explorer / Web Client

Why two protocols? ESP-NOW is fast, consumes very little power and does not require a router connection, which suits a battery node that wakes up and sleeps. MQTT is the standard protocol for cloud communication, and TLS encrypts the connection to the broker.

# 4. Hardware Components

| Component | Qty | Purpose |
| --- | --- | --- |
| ESP32 DevKit | 2 | Board A (Sensor Node) and Board B (Gateway) |
| DHT11 sensor | 1 | Temperature and humidity (GPIO4) |
| SSD1306 OLED | 1 | Display (I2C address 0x3C) |
| External LED | 1 | Alert indicator (GPIO27) |
| Passive buzzer | 1 | Alert sound (GPIO25) |
| Breadboards, jumper wires, USB cables | – | Wiring and power |

Note: the project specification mentions both DHT22 and DHT11. This implementation uses a DHT11. Switching to a DHT22 only requires changing the DHTTYPE definition in the code.

# 5. Wiring

## 5.1 Board A – Sensor Node

| Connection | Pin |
| --- | --- |
| DHT VCC | 3.3V |
| DHT DATA | GPIO4 (10 kΩ pull-up to 3.3V if the module has no built-in resistor) |
| DHT GND | GND |

## 5.2 Board B – Gateway

| Connection | Pin |
| --- | --- |
| OLED SDA | GPIO21 |
| OLED SCL | GPIO22 |
| OLED VCC / GND | 3.3V / GND |
| Buzzer (+) | GPIO25 |
| LED (+) through 220 Ω resistor | GPIO27 |
| LED (−) and Buzzer (−) | GND |

# 6. Software Environment and Libraries

- Arduino IDE with ESP32 board package (Core 3.x).

- DHT sensor library and Adafruit Unified Sensor.

- Adafruit SSD1306 and Adafruit GFX.

- PubSubClient (MQTT client).

- Built-in libraries: WiFi, WiFiClientSecure, esp_now, esp_wifi.

# 7. Packet Structure

The data is sent as a compact binary structure (packed), 14 bytes in total. The same definition is used on both boards.

| Field | Type | Size | Description |
| --- | --- | --- | --- |
| nodeId | uint8_t | 1 | Sensor node identifier |
| bootCount | uint32_t | 4 | Number of boots (stored in RTC memory) |
| temperature | float | 4 | Temperature in °C |
| humidity | float | 4 | Relative humidity in % |
| alert | uint8_t | 1 | 1 = alert, 0 = normal |

# 8. Sensor Node Operation (Board A)

- Wake up from Deep Sleep and increment bootCount (kept in RTC memory using RTC_DATA_ATTR).

- Wait 2 seconds for the sensor to stabilize, then read temperature and humidity.

- Check local thresholds: TEMP_LIMIT = 30 °C and HUM_LIMIT = 70 %. If either is exceeded, set alert = 1.

- Start WiFi in STA mode and fix the channel to WIFI_CHANNEL = 6 (same channel as the router).

- Initialize ESP-NOW, add the Gateway as a peer using its MAC address, and send the packet.

- Wait up to 500 ms for the send callback (Delivered / Failed).

- Enter Deep Sleep for 30 seconds using the timer wake-up.

# 9. Gateway Operation (Board B)

- Connect to WiFi, initialize ESP-NOW and register the receive callback.

- Connect to HiveMQ Cloud on port 8883 (TLS) and publish the birth message.

- The receive callback only stores the packet and raises a flag, keeping it short. The actual processing is done in loop().

- When new data arrives: print it to Serial, update the OLED, apply the local decision and publish JSON to MQTT.

- Local decision: if alert = 1, turn on the LED and sound the buzzer (2000 Hz for 500 ms); otherwise turn the LED off.

- If the MQTT connection drops, loop() reconnects automatically.

# 10. MQTT Details

| Item | Value |
| --- | --- |
| Broker | HiveMQ Cloud (Serverless, Free plan) |
| Port | 8883 (TLS) |
| Client ID | esp32-gateway |
| Data topic | iot/node1/data |
| Status topic | iot/gateway/status |
| Birth message | online (retained) |
| Last Will (LWT) | offline (QoS 1, retained) |

Example data message:

{"node":1,"boot":3,"temp":22.6,"hum":46.0,"alert":0}

LWT mechanism: when the Gateway disconnects unexpectedly (for example a power cut), the broker automatically publishes "offline" on the status topic after the keep-alive period expires. When the Gateway starts again, it publishes "online".

# 11. HiveMQ Cloud Setup

- Create an account at console.hivemq.cloud and create a Serverless (Free) cluster.

- Copy the Cluster URL (used as the host) and the TLS port 8883.

- From Access Management, create credentials (username and password) with the Publish and Subscribe permission.

- Test the connection from the Web Client by subscribing to # and publishing a test message.

- Enter the host and credentials in the Gateway code. Messages then appear in the Web Client or MQTT Explorer.

# 12. Testing and Results

| Test | Expected result |
| --- | --- |
| Start the Gateway | Serial shows WiFi OK, then MQTT OK |
| Start the Sensor Node | Boot #1, then Delivered, then Going to sleep |
| After 30 seconds | Boot #2 (confirms RTC memory and Deep Sleep work) |
| Data delivery | Serial line RX node=1..., OLED updated, JSON visible in MQTT |
| Alert test | Raise the temperature or lower the limit: LED and buzzer activate, alert:1 is published |
| LWT test | Cut power to the Gateway: offline appears on the status topic |

# 13. Challenges and Solutions

| Challenge | Cause | Solution |
| --- | --- | --- |
| Send status "Failed" | ESP-NOW and WiFi share one radio, so both must use the same channel; the gateway MAC could also be wrong | Read the real MAC and channel from the Gateway and fix the channel to 6 on the Node |
| Delays if MQTT is handled in the callback | The callback runs in the WiFi task context | Store the data and set a flag; process in loop() |
| Boot count lost after sleep | Normal RAM is cleared in Deep Sleep | Use RTC_DATA_ATTR |

# 14. Requirements Compliance

| Requirement | Status |
| --- | --- |
| Read temperature and humidity | Done |
| Local threshold check and alert flag | Done |
| Binary packet over ESP-NOW | Done |
| Deep Sleep (30 s timer wake) | Done |
| Boot count in RTC memory | Done |
| Receive and decode packet on Gateway | Done |
| Continuously update OLED | Done |
| Publish to MQTT over TLS | Done (certificate not yet verified) |
| MQTT LWT and birth messages | Done |
| LED and buzzer on alert | Done |

# 15. Limitations and Future Improvements

- Replace setInsecure() with setCACert() to verify the broker certificate (ISRG Root X1).

- Encrypt ESP-NOW traffic using PMK/LMK keys.

- Add a sequence number or checksum to the packet.

- Support multiple nodes using nodeId and one topic per node.

- Use an RGB LED (green = normal, red = alert) and build a dashboard with Node-RED or Grafana.

# Appendix A: Sensor Node Code (Board A)

Credentials are not included in this document.

#include <WiFi.h>

#include <esp_now.h>

#include <esp_wifi.h>

#include <DHT.h>

#define DHTPIN 4

#define DHTTYPE DHT11

#define WIFI_CHANNEL 6

#define SLEEP_SEC 30

#define TEMP_LIMIT 30.0

#define HUM_LIMIT 70.0

uint8_t gatewayMAC[] = {0x44, 0x1D, 0x64, 0xF3, 0x3B, 0x04};

typedef struct __attribute__((packed)) {

uint8_t  nodeId;

uint32_t bootCount;

float    temperature;

float    humidity;

uint8_t  alert;

} Packet;

RTC_DATA_ATTR uint32_t bootCount = 0;

DHT dht(DHTPIN, DHTTYPE);

volatile bool sendDone = false;

void onSent(const uint8_t *mac, esp_now_send_status_t status) {

Serial.println(status == ESP_NOW_SEND_SUCCESS ? "Delivered" : "Failed");

sendDone = true;

}

void setup() {

Serial.begin(115200);

bootCount++;

Serial.printf("Boot #%u\n", bootCount);

dht.begin();

delay(2000);

float t = dht.readTemperature();

float h = dht.readHumidity();

if (!isnan(t) && !isnan(h)) {

Packet p;

p.nodeId = 1; p.bootCount = bootCount;

p.temperature = t; p.humidity = h;

p.alert = (t > TEMP_LIMIT || h > HUM_LIMIT) ? 1 : 0;

WiFi.mode(WIFI_STA);

esp_wifi_set_channel(WIFI_CHANNEL, WIFI_SECOND_CHAN_NONE);

if (esp_now_init() == ESP_OK) {

esp_now_register_send_cb(onSent);

esp_now_peer_info_t peer = {};

memcpy(peer.peer_addr, gatewayMAC, 6);

peer.channel = WIFI_CHANNEL;

peer.encrypt = false;

esp_now_add_peer(&peer);

esp_now_send(gatewayMAC, (uint8_t*)&p, sizeof(p));

unsigned long t0 = millis();

while (!sendDone && millis() - t0 < 500) delay(5);

Serial.printf("T=%.1f H=%.1f alert=%d\n", t, h, p.alert);

}

} else {

Serial.println("DHT read failed");

}

esp_sleep_enable_timer_wakeup((uint64_t)SLEEP_SEC * 1000000ULL);

Serial.println("Going to sleep...");

Serial.flush();

esp_deep_sleep_start();

}

void loop() {}

# Appendix B: Gateway Code (Board B)

WiFi and MQTT credentials have been replaced with placeholders.

#include <WiFi.h>

#include <WiFiClientSecure.h>

#include <PubSubClient.h>

#include <esp_now.h>

#include <Wire.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

const char* WIFI_SSID = "YOUR_WIFI_SSID";

const char* WIFI_PASS = "YOUR_WIFI_PASSWORD";

const char* MQTT_HOST = "YOUR_CLUSTER.s1.eu.hivemq.cloud";

const int   MQTT_PORT = 8883;

const char* MQTT_USER = "YOUR_MQTT_USER";

const char* MQTT_PASS = "YOUR_MQTT_PASSWORD";

#define LED_PIN 27

#define BUZZER_PIN 25

typedef struct __attribute__((packed)) {

uint8_t  nodeId;

uint32_t bootCount;

float    temperature;

float    humidity;

uint8_t  alert;

} Packet;

Adafruit_SSD1306 display(128, 64, &Wire, -1);

WiFiClientSecure net;

PubSubClient mqtt(net);

volatile bool newData = false;

Packet rxPacket;

void onReceive(const esp_now_recv_info_t *info, const uint8_t *data, int len) {

if (len == sizeof(Packet)) {

memcpy((void*)&rxPacket, data, sizeof(Packet));

newData = true;

}

}

void mqttConnect() {

while (!mqtt.connected()) {

Serial.print("MQTT connecting...");

// clientId, user, pass, willTopic, willQoS, willRetain, willMessage

if (mqtt.connect("esp32-gateway", MQTT_USER, MQTT_PASS,

"iot/gateway/status", 1, true, "offline")) {

Serial.println("OK");

mqtt.publish("iot/gateway/status", "online", true);  // Birth message

} else {

Serial.printf("failed rc=%d\n", mqtt.state());

delay(2000);

}

}

}

void showOLED(const Packet &p) {

display.clearDisplay();

display.setTextSize(1);

display.setTextColor(WHITE);

display.setCursor(0, 0);  display.printf("Node %d  Boot #%u", p.nodeId, p.bootCount);

display.setCursor(0, 16); display.printf("Temp: %.1f C", p.temperature);

display.setCursor(0, 28); display.printf("Hum : %.1f %%", p.humidity);

display.setCursor(0, 44); display.print(p.alert ? "!! ALERT !!" : "Status: OK");

display.setCursor(0, 56); display.print(mqtt.connected() ? "MQTT: connected" : "MQTT: down");

display.display();

}

void setup() {

Serial.begin(115200);

pinMode(LED_PIN, OUTPUT);

pinMode(BUZZER_PIN, OUTPUT);

Wire.begin(21, 22);

display.begin(SSD1306_SWITCHCAPVCC, 0x3C);

display.clearDisplay(); display.setCursor(0,0);

display.print("Connecting WiFi..."); display.display();

WiFi.mode(WIFI_STA);

WiFi.begin(WIFI_SSID, WIFI_PASS);

while (WiFi.status() != WL_CONNECTED) delay(300);

Serial.printf("WiFi OK, channel %d, MAC %s\n", WiFi.channel(), WiFi.macAddress().c_str());

esp_now_init();

esp_now_register_recv_cb(onReceive);

net.setInsecure();

mqtt.setServer(MQTT_HOST, MQTT_PORT);

mqttConnect();

}

void loop() {

if (!mqtt.connected()) mqttConnect();

mqtt.loop();

if (newData) {

Packet p;

memcpy(&p, (const void*)&rxPacket, sizeof(p));

newData = false;

Serial.printf("RX node=%d boot=%u T=%.1f H=%.1f alert=%d\n",

p.nodeId, p.bootCount, p.temperature, p.humidity, p.alert);

showOLED(p);

if (p.alert) { digitalWrite(LED_PIN, HIGH); tone(BUZZER_PIN, 2000, 500); }

else         { digitalWrite(LED_PIN, LOW); }

char json[160];

snprintf(json, sizeof(json),

"{\"node\":%d,\"boot\":%u,\"temp\":%.1f,\"hum\":%.1f,\"alert\":%d}",

p.nodeId, p.bootCount, p.temperature, p.humidity, p.alert);

mqtt.publish("iot/node1/data", json);

}

}